Rockler CNC Demonstration #14

February 2, 2019

Text on 3D Component Surfaces

Tools used in this Demo - 1/8” and ¼” Ball Nose

Create Design

- Start new file - Fill out job set up

  - Single sided

  - 11 W x 3.5H x 0.75 Thick

  - Choose data point preferences

  - Click OK

- Go to Clip Art Tab

  - Go to “Domes” folder

  - Select “Dish_Dome_30”, double click will center it

  - Go to “3D Tabs” folder

  - Select “Rectangular_0.5”, double click will center it

  - Select “Rectangular_0.25” and drag one to line up with left of the work piece

- Go to drawing tab to resize all 3D Objects

  - Select the Dome

  - Select “Set Selected Object Size”

  - Enter 9” in either width or height and check box to link XY

  - Click Apply and Close

  - Select the Rectangular 0.5” Object

  - Select “Set Selected Object Size”

  - Enter 8” W x 3” H

  - Click Apply and Close

  - Select the Rectangular 0.25 Object

  - Select “Set Selected Object Size”

  - Enter 1” W x 0.75” H

  - Click Apply and Close

- Adjust height and relationship between all 3D Objects

  - Go to Modeling Tab

  - Select the Dome by double clicking to bring up the properties box

  - Change Height to 0.5”

  - Change Base Height to 1/16”

  - Check “Add”

  - Click Close

  - Select the Rectangular Object by double clicking to bring up the properties box

  - Change Height to 3/16”

  - Check “Add”

  - Click Close

  - Locate first 3D tab, got to properties and change to 3/8” thick, locate on left side then copy to the right side

- Create Text Box

  - Go to Modeling Tab

  - Select Rectangular _0.5

  - In 3D Model Tools, select “Create Vector Boundary”

  - Go to Drawing Tab

  - Select the boundary box created in step above

  - Select “Draw Text in Vector Box”

  - Enter “Rockler”

  - Chose Text “Gulim”

  - Check Bold

  - Check Normal Margin

  - Vertical stretch set to “Stretch Characters”

  - Click “Close”

  - Select “Edit text spacing” and add spaces between letters to allow for an 1/4” EM tool

  - IMPORTANT Select Box and create an offset of 1/8”, delete original, TJ to explain this

- You should have something like the above picture and are now ready to make TOOL PATHS

Create TOOL PATHS

- Set properties

  - Tool clearance to 1”

  - Set datum to lower left

- Use 3D Roughing tool path

  - Select the border box

  - Select 3D Roughing tool path

    - Select 1/4” Ball Nose and enter depths and speeds for your machine

    - Check “Selected Vectors”

    - Enter a boundary offset of 0.25”

    - Enter a machine allowance of 1/32”

    - Select Z Level, Raster X, Profile None

    - Change Name to Roughing Clear

    - Click Calculate

- Use 3D Finishing tool path

  - Select the border box

  - Select 3D Finishing tool path

    - Select 1/4” Ball Nose and enter depths and speeds for your machine (to make a nice smooth finish on the top of the letters, either use a small step over number or sand smooth when done)

    - Check “Selected Vectors”

    - Enter a boundary offset of 0.125”

    - Select Offset Conventional

    - Change Name to Finish Surface

    - Click Calculate

- Use the “Pocket” tool path

  - Select both the text and border box

  - Select Pocket tool path

    - Set start depth to 0”

    - Set cut depth to 3/16”

    - Check “show advanced toolpath options”

    - Select 1/8” Ball Nose and enter depths and speeds for your machine

    - Check “Use Larger Area Clearance Tool”

    - Select 1/4” Ball Nose and enter depth and speeds for your machine

    - Select offset / conventional

    - VERY IMPORTANT Click box for “Project toolpath onto 3D model”

    - Change name to Pocket Text

    - Click Calculate

  - You will notice 2 toolpaths were made, one for each tool size

- Save above Tool Paths to USB drive

  - Add tool description to file name, for pendant machines make first 5 characters meaningful

  - Save ENTIRE project file

- DONE – ready to cut

Set up wood to be cut

- Make sure properly secured

- JOG create 0,0,0

- Run file for 3D Rough (pre run before Demo)

- Run file for 3D Finish (pre run before Demo)

- Run file for Text Pocket Clear

- Run file for Text Pocket

Question?

Ideas for next Demo…